STANDARD TUR55

Trusted Unified Regulation (TUR55) for executing blockchain commands on IoT devices.

## Version: v1.0

## Status: Fixed

## Date of fixation (UTC): 10.04.2026

## Author

- Alias/Identifier: _NOMOS-H_

- Public Key Fingerprint: 2D0C4604CCF007C1AFDDADBBDBB49B9C183EB014__

## Content Integrity

- Document Hash (SHA-256): ____________________

- IPFS CID: ____________________

- Blockchain Anchor (tx / block): ____________________

TUR55 is an open standard for transmitting commands from blockchain to IoT devices. It solves the problem of the lack of a unified protocol for executors (analogous to oracles, but for execution)

1. Scope

This standard describes a unified protocol for transmitting and executing commands from smart contracts to resource-constrained devices (ESP32, STM32, Raspberry Pi Pico, and others) running MicroPython.

The protocol provides:

- blockchain event monitoring;

- formation of an encrypted executable package;

- package delivery via any communication channels;

- secure script execution in an isolated environment (sandbox);

- optional execution confirmation.

2. TUR55 Protocol Layers

The standard architecture is divided into four logical layers, similar to the OSI/TCP/IP model.

Layer 1: Event Layer (Blockchain Event Layer)

Responsible for recording the occurrence of a condition in the smart contract.

- The smart contract generates an event when a condition occurs (payment, expiration, DAO voting, etc.).

- The event contains:

- `user_wallet` — payer or initiator address;

- `amount` — amount or limit (optional);

- `expires` — command validity time;

- `device_id` — target device identifier;

- `metadata` — additional data (action type, parameters).

Layer 2: Executor Layer (Executor Layer)

A component (service, bot, relay) that listens to the blockchain and forms an executable package.

- Subscribes to smart contract events.

- Extracts data and forms a JSON envelope.

- Embeds data into a Python script template.

- Encrypts the resulting script.

- Transfers the encrypted package to the delivery layer.

Layer 3: Delivery Layer (Delivery Layer)

Responsible for transferring the package to the target device.

- Supported channels:

- MQTT;

- HTTP/HTTPS;

- Bluetooth (BLE);

- LoRaWAN;

- QR-code / NFC;

- physical media (SD card, USB).

- The package is delivered as a binary file or base64 string.

Layer 4: Execution Layer (Execution Layer)

Executed on the IoT device in a secure environment.

- Package decryption using hardware or embedded key.

- Checking the validity period (`expires`).

- Parsing JSON into a Python script.

- Script execution in a sandbox with time, memory, and system access restrictions.

- Optional sending of execution confirmation.

3. Data Formats

3.1. Smart Contract Event (EVM example)

```solidity

event ExecutorTriggered(

address indexed userWallet,

uint256 amount,

uint256 expires,

bytes32 deviceId,

bytes metadata

);

```

3.2. JSON Envelope (formed by executor)

```json

{

"tur55_version": "1.0",

"command_id": "0x7e5f8a3c...",

"device_id": "esp32_001",

"source": {

"smart_contract": "0x1234...",

"transaction_hash": "0xabcd...",

"block_number": 12345678

},

"execution": {

"user_wallet": "0xUserWallet...",

"amount": 50.5,

"expires": 1741500000

},

"custom_data": {

"action": "unlock",

"duration": 30

}

}

```

3.3. Python Script Template with Data Substitution

```python

# === TUR55 EXECUTION CONTEXT ===

import machine

import time

# Data from blockchain (substituted by executor)

user_wallet = "$USER_WALLET"

amount = $AMOUNT

expires = $EXPIRES

custom_data = $CUSTOM_JSON

# === USER CODE ===

# (written by device developer)

def main():

print(f"Executing for {user_wallet}, amount: {amount}")

if custom_data.get("action") == "unlock":

machine.Pin(2, machine.Pin.OUT).value(1)

time.sleep(custom_data.get("duration", 30))

machine.Pin(2).value(0)

if __name__ == "__main__":

main()

```

3.4. Encrypted Package (structure)

```

[HEADER: TUR55v1]

[ENCRYPTED_SESSION_KEY] (encrypted with device public key)

[ENCRYPTED_SCRIPT] (AES-256-GCM)

[SIGNATURE] (ECDSA / Ed25519 signature executor)

```

4. Cryptographic Requirements

| Function | Algorithm | Purpose |
| --- | --- | --- |
| Script Encryption | AES-256-GCM | Command Confidentiality |
| Session Key Encryption | ECIES or RSA-OAEP | Key Transfer to Device |
| Package Signature | Ed25519 or ECDSA secp256k1 | Source Authentication |
| Hashing | SHA3-256 or Blake2s | Integrity |
| Device Key | Stored in secure memory (eFuse, Secure Element) | Device Identification |

5. TUR55 Command Lifecycle

1. Event Occurrence in the smart contract (payment, license expiration, voting, etc.).

2. Executor (service) receives the event, forms JSON, embeds data into Python script.

3. Encryption of script and session key, package signing.

4. Delivery of package to device via any available channel (MQTT, BLE, QR, LoRa).

5. Device receives the package, verifies signature, decrypts, checks `expires`.

6. Execution of script in sandbox:

- execution time limit (timeout);

- memory limit;

- prohibition of unsafe operations (file system, network, unless permitted).

7. Optionally: the device forms a confirmation, signs it, and sends it back to the executor or directly to the blockchain.

6. Security and Isolation (Sandbox)

- Forbidden MicroPython Modules: `os`, `socket`, `urandom` (if not required).

- Limitations:

- Maximum execution time: 5–30 seconds (configurable).

- Maximum memory: 16–64 KB (depending on device).

- GPIO access only to predefined pins.

- Hardware Protection (recommended):

- Using Secure Element (ATECC608, SE050) for key storage.

- Hardware AES acceleration.

7. Compatibility and Transports

TUR55 is not tied to a specific blockchain or communication channel. Supported:

- Blockchains: Ethereum, Polygon, BSC, IOTA (Tangle), Solana, Cosmos and others.

- Transports: MQTT, HTTP, BLE, LoRaWAN, QR, NFC, physical media.

8. Usage Example: Software License

1. User pays for subscription in the smart contract.

2. Executor forms a script that disables API access after the expiration date.

3. Encrypted package is delivered to the MicroPython server.

4. Server executes the script in sandbox, blocking the API key.

5. Optionally sends confirmation to the blockchain.

9. TUR55 Compliance Levels

| Level | Capabilities |
| --- | --- |
| TUR55/Bronze | Package reception, decryption, execution, UART log |
| TUR55/Silver | + Result signature, sending via MQTT/HTTP |
| TUR55/Gold | + Secure boot, secure key storage, remote attestation |

10. Conclusion

TUR55v1 establishes a unified standard for transmitting commands from blockchain to the physical world, solving the fragmentation problem existing among executors (analogous to oracles, but for execution). The protocol is blockchain-agnostic, transport-independent and ensures secure code execution in a constrained environment.

11. License Terms

11.1. Standard Status

Standard TUR55v1 (Trusted Unified Regulation) is an open public standard (Open Public Standard).

Any person or organization has the right to:

- use the specification to implement compatible devices, software, and services;

- distribute and cite the document in whole or in part;

- create forks and modifications of the protocol provided that authorship is preserved and a link to the original is provided.

11.2. Mandatory Usage Terms

When using the standard in any way (including implementations, forks, derivative works, commercial products) mandatory:

1. Preserve authorship attribution:

-"Based on TUR55 Standard (Trusted Unified Regulation), author: NOMOS-H"

2. Include a link to the original specification:

-"Original: [URL GitHub / IPFS / Arweave]"

3. Do not remove or hide built-in royalty mechanisms (if implemented in code), unless otherwise agreed with the author.

11.3. Commercial Use

Commercial use of the standard is permitted without royalty payments, but:

- It is prohibited to pass off derivative standards or implementations as "official TUR55" without explicit indication of modification.

- It is prohibited to use the trade name "TUR55" or its stylization in the names of commercial products without written permission from the author.

11.4. Exceptions and Limitations

The author of the standard is not liable for any damage, direct or indirect, arising from the use of this standard or its implementations.

The standard is provided "as is" (AS IS), without any warranties.

11.5. Contact Information

For licensing inquiries, obtaining permission to use the trade name, or entering into commercial agreements:

- Email: tur55@protonmail.com

- GitHub: https://github.com/Tabintaban

- Discord: tur55

# 12. Version Statement

This document represents the first fixed version (v1.0). 
Changes are not permitted.
 Future changes must be introduced only as new versions.

The date of birth of the TUR55 protocol is 10.04.2026 years, the mention of the protocol TUR55 earlier than this date is drafts and preliminary design.

# 13. Declaration of Authorship

I, NOMOS-H, hereby declare authorship and priority
of the content described in this document as of the fixation date stated above.

## Cryptographic Signature

(signature)

End of document